All samples were first oven dried at 70 °C for 72 hours and placed under a vacuum with specific saturation fluids to saturate the effective pore space. Samples were sealed with semi films and silicone gel to prevent dehydration, which could increase impedance dramatically.

The concentration of the NaCl brine for saturation was prepared at 0.01 mol/L. After saturation, all samples were kept in the same holder with saturation fluids to stabilize and normalize the conductivity (see Table 1 for the stabilized conductivity). The sample holder for the SIP test was built as a modified Wenner array with Ag/AgCl electrodes. SIP data were collected using a Portable SIP meter (Ontash & Ermac, Inc.) over a frequency range from 0.01 Hz to 10k Hz to ensure the datasets can cover the entire pore-size distribution, including macropores. The lower limit of phase shift is about 0.2 mrad. The reference resistor at 10k Ohm is close to the same order of magnitude of the impedance of the samples and was chosen to provide an optimal signal to noise ratio. The measured impedances and phase shifts can be converted to real and imaginary conductivities using a geometry factor equal to the contact surface area of the sample divided by the distance between two potential electrodes (0.0179 m plus the sample height). The value of the  spectra and spectra in 1Hz of frequency is used as the characteristic and   value for the sample.